User’s Manual

SD Card Laser Light Control System

First: Key Operate

○   ○   ○   ○

MODE           DOWN            UP             ENTER

As the figure shown:

Mode is Menu Key, ENTER is ensure key, UP is flip,, DOWN is next. After opening, Machine state is saved when the last shutdown state, This time, Press MODE key to exit the mode selection menu, LCD

First line display “mode select”, The second line display Operate Mode:

Program Table

Key Seek

Audio Mode

Auto Mode

DMX Mode

Slave Mode

System set can set color mode, Audio senor effect, X/Y phase

System Set working file set

Repeat press “MODE” key jump to the working mode you want, press “ENTER” key ensure.

Mode State and operate

- Program Table: LCD first line display “SD Program Show”, Second line display current list of player names, that time, press “UP” and “DOWN” select play list.

- Key Seek: LCD first line display “Show File”, Second line display current play ILDA file document name, that time, press “UP” and “DOWN” select ILDA file.

- Audio Mode: LCD first line display “Audio show”, Second line display System Development Company Name “Aurora Studio”, that time, play card interior integrated program, the rhythm and the sound effect changed by the environment

- Auto Mode: LCD first line display “ Auto Show” , Second line display System Development Company Name “Aurora Studio”, that time, play card interior integrated program.

- DMX Mode: LCD first line display current DMX address, press “UP” address add 1, press “Down” address Subtract 1, continuously press “UP” or “DOWN” will fast continuously change. When adjust to required “DMX” address, press “ENTER” ensure.

| 6 | zooming | 0-10 | Without zooming |
| --- | --- | --- | --- |
|  |  | 11-100 | Manually adjust the size |
|  |  | 101-150 | Enlarge |
|  |  | 151-200 | Shrink |
|  |  | 201-255 | Cycle Shrink |
| 7 | Around X-axis rotating | 0-10 | Without rotating |
|  |  | 11-110 | Manually adjust |
|  |  | 111-255 | Auto rotating |
| 8 | Around Y-axis rotating | 0-10 | Without rotating |
|  |  | 11-110 | Manually adjust |
|  |  | 111-255 | Auto rotating |
| 9 | Around Center rotating | 0 | Without rotating |
|  |  | 1-180 | Manually adjust |
|  |  | 181-217 | Auto clockwise rotating |
|  |  | 218-255 | Auto counterclockwise rotating |
| 10 | Drawing | 0-10 | Without change |
|  |  | 10-74 | Manually adjust rotating |
|  |  | 75-104 | Auto drawing（Add） |
|  |  | 105-144 | Auto drawing（Subtract） |
|  |  | 145-184 | Auto cycle drawing |
|  |  | 185-224 | Auto end to end drawing(Add) |
|  |  | 225-255 | Auto end to end drawing（Subtract） |
| 11 | Waving | 0-9 | Without waving |
|  |  | 10-199 | Waving speed can adjust |
|  |  | 200-255 | Waving width can adjust |
|  | Color |  |  |
| 12 | RGB | 0-16 | White |
|  |  | 17-33 | Red |
|  |  | 34-50 | Green |
|  |  | 51-67 | Blue |
|  |  | 68-84 | Yellow |

6.  Slave Mode: At this time play state according Master mode change to change

- System set: LCD first line display “System Set”, Second line display setting parameters, that time continuously press “UP” or “DOWN” select required setting parameters, then press “ENTER” ensure, “Color Mode Set” color mode setting, “Audio Set” audio effect setting, “Phasic Set” phase setting.

- Folder set: LCD first line display “Folder Select”, Second line display folder name, press “UP” or “Down” to required folder , press “ENTER” ensure.

Second: Channel Description

| Channel Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel |  | Value | Control Description |  |
| 1 | Mode Select | 0-73 | Light Shut Off |  |
|  |  | 74-110 | PRG Mode |  |
|  |  | 111-147 | ILD Mode |  |
|  |  | 148-184 | Audio |  |
|  |  | 185-221 | Auto |  |
|  |  | 222-255 | Manually Mode |  |
| 2 | Graphic/Folder Select | 0-255 | Manually Mode | PRG/ILD Mode |
|  |  |  | Graphic change, per 2 value is one graphic | Folder select |
| 3 | Strobe/Folder Select | 0-10 | Without Strobe | Play file select |
|  |  | 11-199 | Auto Strobe |  |
|  |  | 200-255 | Audio Strobe |  |
| 4 | X-axis direction moving | 0-125 | Manually adjust location |  |
|  |  | 126-185 | Auto left and right cycle move |  |
|  |  | 186-225 | Auto jump left and right cycle move |  |
|  |  | 226-245 | Auto without rule jump |  |
|  |  | 246-255 | Audio without rule jump |  |
| 5 | X-axis direction moving | 0-125 | Manually adjust location |  |
|  |  | 126-185 | Auto up and down cycle move |  |
|  |  | 186-225 | Auto jump up and down cycle move |  |
|  |  | 226-245 | Auto without rule jump |  |
|  |  | 246-255 | Audio without rule jump |  |
| 12 | RGB | 85-101 | Violet |  |
|  |  | 102-118 | Cyan |  |
|  |  | 119-135 | White, red, green, blue four color partition |  |
|  |  | 136-152 | Blue, yellow, violet, cyan four color partition |  |
|  |  | 153-169 | White, red, green, blue, yellow, violet, cyan, seven color partition |  |
|  |  | 170-186 | White, red, green, blue four color flow |  |
|  |  | 187-203 | Blue, yellow, violet, cyan four color flow |  |
|  |  | 204-220 | Blue, yellow, violet, cyan four color flow |  |
|  |  | 221-237 | According breakpoint partition display |  |
|  |  | 238-255 | Audio discolor |  |
|  | RGY | 0-27 | White |  |
|  |  | 28-55 | Red |  |
|  |  | 56-83 | Green |  |
|  |  | 84-111 | Yellow |  |
|  |  | 112-139 | Red, green double color partition |  |
|  |  | 140-167 | White, red, green, blue four color partition |  |
|  |  | 168-195 | Red, green double color flow |  |
|  |  | 196-223 | White, red, green, blue four color flow |  |
|  |  | 224-255 | Audio discolor |  |
| 13 | Display Mode | 0-63 | Normal display |  |
|  |  | 64-127 | Highlights display |  |
|  |  | 128-191 | Partition display |  |
|  |  | 192-255 | Point display |  |